LIC-Mini Project

Date:- 06/11/2023

By:- Monday Batch 1

Team Members:-

- Harshit Malik – 107121039

- Kshitij Kanade – 107121045

- Mayank Pachauri - 107121055

Task

Designing an Analog PID Controller

Problem Statement:-

Design and fabricate an analog PID controller circuit for the plant with following requirements:

i. The gain of the plant is 0.5.

ii. The controller parameters are Kp=1; Ki=10 and Kd=3.

iii. The output of the plant has to be maintained at 2V.

iv. Self-power generation circuit for signal conditioning circuit (Assume dc input voltage variation is from 15V to 30V).

(NOTE: Don't use external power supply in the circuits.)

Abstract:-

In this project, we're designing and building an Analog PID controller circuit using operational amplifiers to meet specific plant requirements. The PID controller will ensure stable and efficient control, with a target output of 2V. We'll also incorporate a self-power generation circuit for operation within a 15V to 30V voltage range. The project combines theory, practical implementation, and testing of proportional, integral, and derivative components. Ultimately, it aims to create a fully functional Analog PID controller, enhancing control systems in various industrial applications.

Equipment Needed:-

| S No. | Equipment | Ratings | Quantity |
| --- | --- | --- | --- |
| 1 | OP-AMP | LM741CN | 7 |
| 2 | Voltage Regulator | LM7815T,LM7915 (If required) | 1, 1 |
| 3 | Resistors | 100 kΩ,10 kΩ, 20 kΩ, 150 kΩ, 1 kΩ, 300 kΩ, 130 kΩ | 17,1,1, 1,1,1 1 |
| 4 | Capacitor | 10µF | 2 |
| 5 | Rheostats | 100 kΩ | 3 |
| 6 | Breadboard, PCB Board, DSO, Multimeter, Probes, Connecting Wires | --- | As required |

Thought Process Behind Circuit Development:-

- Initially we designed the proportional amplifier, integrator, and differentiator using Inverting Amplifier circuit and fixed the corresponding gains.

- In the second step, we added all these signals using OpAmp as inverting summer and passed the signal through the plant to get output at constant 2 Volts.

- This output was also fed back to the inverting subtractor which compared the Input and output voltage and generated the error signal.

- This error signal in turn was fed to the PID controller block.

- The self-power generation circuit takes in a dc input between 15-30V and is passed through IC7815 voltage regulator which gives constant +15V supply; the -15V supply can be generated either by passing this signal though an inverting amplifier, or using separate IC7915 which are used to power the OpAmps.

- A simple resistive voltage divider circuit can be used to generate the reference voltage required just before the PID controller block.

- Else to observe the changes on setting the fixed point at 2V, pass a square wave with maximum amplitude of 2V and observe the changes in output voltage like overshoot, etc.

Issues faced :-  Generating -15V for supplying to OpAmps, reference voltage generation.

Basic Circuit Diagram of PID Controller:-

TRANSFER FUNCTION REPRESENTATION:-

Sometimes it is useful to write the PID control equation in Laplace transform form which is given by: 	G(s) =	kp +ki/s + kd s = kd*s2+kp*s+kis

Block Diagram and Results of the circuit:-

Corresponding Waveform-

MATLAB Simulation of Actual Circuit using OpAmps:-

Corresponding Waveform-

Self-power generation circuit for signal conditioning:-

Corresponding Waveform-

PCB Simulation using easyeda:-

PCB Schematic-

Final Circuit Diagram including Self-powering circuit-

Breadboard Implementation of Entire Circuit:-

-------------Supply + regulation

--------------------------------PID

Zoomed image------------------------

----------------------Waveform at 2V change

Conclusion:-

It was found that the proportional controller reduces the transients to an appreciable extent and thus, should have a high value. The derivative controller acts on the rate of change of input and thus converts the triangular wave to a square wave. It is very sensitive to changes or variations in the input. As far as the integral controller was concerned, it had a slow rise and decay time making the system sluggish. The output waveforms were found almost as expected and thus, the analog pid controller was fabricated finally.

Applications:-

- Industrial Furnaces and HVAC Systems:

- PID controllers regulate temperature in industrial furnaces, ensuring efficient and consistent heat.

- They maintain optimal temperatures in heating, ventilation, and air conditioning (HVAC) systems for comfort and energy efficiency.

- Robotics and Conveyor Systems:

- Precise motor control in robotics for accurate movements and task execution.

- Conveyor systems use PID control to manage conveyor belt speed and product positioning.

- Manufacturing Processes:

- PID controllers regulate chemical processes in manufacturing, ensuring product quality and consistency.

- They control variables like pressure, flow rates, and composition in chemical plants.

- Aircraft Flight Control:

- Essential for maintaining stable flight by adjusting control surfaces and engine thrust.

- Ensures passenger safety and a smooth flying experience.

- Renewable Energy Systems:

- Used in solar trackers and wind turbine controls to optimize energy production.

- PID control adapts to changing environmental conditions.

In summary, PID controllers play a crucial role in a wide range of applications, offering precise control and stability in diverse fields, from industrial processes to renewable energy systems and healthcare devices.

Thank You